Here is the step-by-step process to set it up and initiate it.

1. Initial In-Vehicle Setup

Before you can use the app to call your truck, you have to enable the feature from the main touchscreen while the truck is in Park.

- Go to Controls > Self-Driving (or Autopilot) > ASS (Actually Smart Summon).

- Read the safety prompt and toggle it to Yes.

- (Optional) You can also toggle Standby Mode here. It keeps the truck's cameras and computer warmed up so it responds instantly in the app, though it draws a bit more battery.

2. Pre-Check Prerequisites

To prevent the app from throwing an error or graying out the buttons, ensure:

- You are on private property (like a parking lot or driveway). The system is geofenced to deactivate on public roads.

- You are within 279 feet (85 meters) of the truck.

- You have a clear, direct line of sight to the vehicle.

- All doors, the vault (tonneau cover), and the frunk are fully closed.

3. Initiating via the Tesla App

Open the latest version of the Tesla mobile app on your phone and follow these steps:

1.Open the Summon Menu:Tesla App.Tap the Summon option on your app's main screen. A map overlay will appear showing a blue circle (your max operating boundary) with a blue dot representing your location and a red icon for your truck.

Select your target destination:

- Come to Me: Tap this option to target your phone's current GPS location.

- Go to Target: Tap the crosshair icon, drag the map to drop a pin exactly where you want the truck to park itself, and select it.

Press and continuously hold down the chosen action button. The Cybertruck's hazard lights will flash briefly, the side mirrors will fold in, and it will shift into gear.

Keep holding the button as the truck autonomously maneuvers out of its spot and navigates to the target destination at a safe, low speed.

Crucial Safety Note: To stop the Cybertruck at any exact moment, simply lift your finger off the phone screen. The vehicle will immediately brake and pause its route. Once it successfully reaches its destination, it will shift into Park and flash its hazard lights to let you know it's done.